# Faculty Advisor Team Update: Electrical Team Meeting Notes

Project: Universal Plastic Shredder V2.0

Date: 02/06/26
Time: 11:00AM -12:00PM
 Location / Call Link: Online

## Attendees

### Electrical Team

- Bao Nguyen

- Yaqoub Rabiah

- Fearghus Tyler

- Fox Kang

### Mechanical Team

- Adan V.

- Baily Lindemann

- Derek

### Sponsor (Ichor Systems)

- Bridget Lannigan – Sponsor, Mechanical Engineer

## Technical Notes / Discussion

Mechanical Team

- Coupler Issue
 Baily Lindemann is struggling with the coupler design for the shredder.
 Bao recommended using a Lovejoy coupler.

- Keyway Machining
 Greg suggested purchasing a specialized keyway cutting tool.
 The concern is cost, since the tool would only be used twice.
 The team would prefer to manually scrape the keyway instead of buying the tool.

- Shaft Rubbing and Wear Prevention
 To prevent rubbing, McMaster-Carr black phosphate–coated components were discussed.
 The coating provides lubrication and reduces wear even if there is slight shaft movement or resistance.
 If the shaft is pushed toward the chassis, the coating should prevent damage.
 Components will be placed just before the bearings to reduce wear.

- Blade Configuration
 The shredder uses four identical blades with different orientations.
 For a double-sided shredder, blade orientations are staggered so blades do not contact each other simultaneously.
 Blades are rotated by 15 degrees, resulting in a total of 12 effective cutting positions without blade interference.
 A 1/16-inch gap is maintained between blades to prevent rubbing and premature wear.

- Open Issue
 Gavin was not present, so the final blade angle could not be confirmed.

- Cost and Gearbox
 Current BOM total cost is $3,323.
 A 180:1 gearbox is being considered as an option.

Electrical team

- Request a 20x20” Box for the enclosure

## Action Items / Team Tasks

- Fox

  - Research and design a circuit for capacitive touch sensor

- Yaqoub

  - Program and update the ladder logic with correct deadman switch behavior

  - Lights

  - Possible analog RPM control

- Bao

  - Wire the contactor and buttons that are available

- Fearghus

  - Finish schematic

Ichorsystem engineers:

Stonefire, Ben Mechanical engineering.

Lannigan, Bridget

-Recommended to have wireless connection

-Jam count to tell the operator to put smaller pieces etc

-Concerns about starting and stopping the machine wirelessly

-Sendcut send for sheet metal cutting

-need 1.5hp minimum, but want more margin to shred 1 inch stock

-3000 target, 5000 budget

-Ideally use corporate card to pay for things, but can be reimbursed if you buy anyways.

-Fox Khang Industry standard need help

-Ycobu found the VFD 3hp 120-230V

-Nothing to do this week

Thigns for the team to do

FOx-

working bom on safety parts

Yaq

VFD testing

Bao:tbd

Feaghustbd