# Electrical Team Meeting Notes

## Purpose

Align on VFD selection, PLC platform, power distribution, and safety integration for the shredder system.

## Attendees

- Ohara

- Nicholas

- Alain Coulombe

- Ben Stonefire

- Bridget Lannigan

## 1. VFD Selection

- Proposed: Siemens Micromaster 440

- Need to confirm:

  - Motor HP rating

  - Input voltage (single-phase or three-phase)

  - Output current

  - Required control method (analog input, Modbus, etc.)

Open Question:

- What is the target shredder shaft RPM?

  - Mechanical team must confirm required speed range.

  - This determines VFD sizing and programming.

## 2. 24V Control Power

- Use industrial 24V DC power supply

  - Meanwell or equivalent

- Confirm total current draw:

  - PLC

  - Sensors

  - LED indicators

  - Safety circuits

## 3. Indicator Lights

- Green: 1091M5-24V

- Red: WL-1091M1-24V

- Confirm panel mounting and wiring scheme.

## 4. Emergency Stop

- Industrial E-Stop with yellow background plate

- Confirm:

  - Normally closed wiring

  - Hardwired to contactor/VFD enable

  - Not software-dependent

## 5. Safety Integration

- Sensor failure concern raised by Ohara

- Electrical and mechanical safety must be integrated

- Consider:

  - Interlocks

  - Guard switches

  - Overcurrent protection

  - Dust environment impact on electronics

Note: PLC logic alone is not sufficient for safety. Critical shutdown paths must be hardwired.

## 6. PLC Platform / Programming

Discussion Points:

- What PLC are we finalizing?

  - Alteri?

  - Albec?

  - Siemens?

- Does it support CODESYS?

- If using Siemens PLC, will we program in TIA Portal instead?

Need to standardize platform before moving forward with ladder logic or structured text development.

## Action Items

- Confirm shredder speed requirements

- Finalize PLC platform

- Size 24V power supply based on load calculation

- Verify VFD rating matches motor

- Define safety architecture (hardwired vs PLC monitored)