# I'm So Shredded Inc. — Ichor Systems Shredder V2 Control System Test Plan

2026/05/01

# Overview

The test plan is a Standard Operating Procedure (SOP) that objectively maps your current system to its requirements (as found in your Project Description Specification (PDS)). More specifically, after running through your test plan, you should have a list of all of the requirements that your project does and doesn’t meet. The test plan can be as formal as the ECE411 table attached at the bottom of this document or as simple as a bulleted list of steps to step through. Broadly speaking, there are two kinds of testing:

# Step 1: Top Down Testing

The purpose of this test plan is to validate that the complete shredder system operates correctly and safely against its requirements. This will be done by exercising the full control chain from power-on to normal operation and shutdown.

# Step 2: Bottom Up Testing

This portion of the test plan will go over each segmented case for our full system.

Test case 1: Verify 120VAC input is correctly converted by the VFD

Test case 2: Verify the contactor energizes and de-energizes correctly under control commands

Test case 3: VFD outputs to the motor across a range of frequencies

Test case 4: Verification of safety devices( E-stop, dead man switches) independently function

Test case 5: Verify the HMI correctly sets the frequency and shows the system+telemetry information

# Caveat

We’re playing fast and loose with the word “testing” here. Technically, like in ECE411, we should be doing verification against design specification  and validation against user requirements, but we don’t need to get that formal for this test plan so we can lump it all under “testing”.

# Things to remember

- Write your test plans as if someone who is NOT on your team is going to run the test. And, for special extra bonus points, actually get someone who isn’t on your team to run your tests. It will really open your eyes about what assumptions you’re making about your system.

- Don’t forget about equipment needed, setup, and teardown. Again, someone who isn’t familiar with your system should be able to run these tests.

# Top Down Test Plan v1.0

| Date of Test: | May 1, 2026 |
| --- | --- |
| Tester: | Low level ee |

## Purpose

To test the entire electrical control system. Validate the function of the PLC, VFD, and HMI.

## Equipment Needed

- System Equipment

  - DirectLogic 205 PLC rack

  - EA1-T4CL HMI

  - HY02D211B-T VFD

  - 120VAC Wall Power

  - Mitsubishi SN35 Contactor

  - Estop

  - Forward and Reverse button (Seperate)

  - Deadman switch (2 total)

  - CTSI Module (Capactive touch safety Module Interface)

  - Laptop

  - Gearbox and shredder

- Test Equipment

  - Hotdog sausage

  - Multimeter

  - Crowbar for jamming

  - Testa material: 0.25” and 0.5” thick PVC or PP plastic scrap

  - RPM counter goal 70-90rpm

- Other Equipment

  - Level 5 NIJ body armor

  - Safety Glasses

  - Fire extinguisher

## Pre-Test Setup

- Confirm shredder hopper is empty and the blade chamber is clear of all material and obstructions.

- Engage (press) E-stop to a safe state. Confirm both deadman switches are released (open).

- Verify all wiring connections (VFD L1/L2, U/V/W, PLC I/O terminals) are tight per wiring diagram.

- Connect laptop to PLC via programming cable. Open DirectSoft; confirm COMM LED = solid green.

- Multimeter: E-stop contacts = open circuit (engaged) / <1 ohm (released). Check both.

- Don PPE (Level 5 NIJ body armor + ANSI Z87.1 safety glasses). Place the fire extinguisher within 3 ft.

## Top Down Test Steps

- Main power initialization

  - Release E-stop (twist-pull). Switch wall breaker ON. Confirm VFD shows 0.0 Hz, no fault codes.

  - Confirm PLC RUN LED = green, ERR = off. Confirm HMI shows READY and setpoint = 0 Hz

- Set HMI frequency = 30 Hz.

  - Engage both deadman switches.

  - Confirm contactor closes (audible click),

  - VFD ramps to 30 Hz, RPM = 70-90.

  - Release one deadman; confirm system stops within 3 sec.

## Post-Test Teardown

- Press E-stop, power button. Switch wall breaker OFF. Wait >= 30 sec for VFD capacitors to discharge.

- Disconnect power cord and PLC programming cable. Inspect blade chamber for debris or damage.

- Return test equipment to storage. Complete and sign the Conclusions section below.

## Top-down Test Plan  Conclusions / Discussion

Record pass/fail for each step above. Note any fault codes, out-of-spec readings, or deviations.

Tester signature: _______________________ Date: _________

# Bottom Up Test Plan v1.0 (Template)

| Test Author: Bao Nguyen |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Name: |  | VFD 120 VAC Input Conversion |  |  | Test ID #: |  |  |  |
|  | Description: |  | Verify the HY02D@11B-T VFD accpets 120 VAC input and produces a stable DC bus voltage. Motor should be disconnected , no load on VFD output |  |  | Type: |  |  | □ white box ✓ black box □ _____________ |
| Tester Information |  |  |  |  |  |  |  |  |  |
|  | Name of Tester: |  |  |  |  | Date: |  |  |  |
|  | HW/SW Version: |  | 1.0 |  |  | Time: |  |  |  |
|  | Setup: |  | System OFF. E-stop engaged. VFD output (U/V/W) disconnected. Multimeter set to AC 600V and DC 500V ranges. PPE worn. |  |  |  |  |  |  |
| T E S T | INPUTS |  |  | EXPECTED OUTPUTS |  | P A S S | F A I L | N/A | Comments |
| 1 | Apply 120 VAC to VFD L1/L2. Measure AC voltage at input terminals |  |  | 108-132 VAC at L1/L2 VFD display turns in. No fault codes. Power LED is solid |  |  |  |  |  |
| 2 | Measure DC bus voltage at VFD DC+/DC- terminals. CAUTION: high voltage use DC 500V range |  |  | DC bus ~160-170 VDC. Stable reading <5V |  |  |  |  |  |
| 3 | Allow VFD to power up for 10 seconds. Read front panel display |  |  | Display shows 0.0 Hz. No fault or alarm codes |  |  |  |  |  |
| 4 | Remove 120VAC, measure DC bus every 10 seconds until. Record de-energize time. |  |  | DC bus bleed to <10 VDC within 60 seconds of power removal. Record:____ seconds. |  |  |  |  |  |
|  | Overall test result: |  |  |  |  |  |  |  |  |

| Test Author: Bao Nguyen |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Name: |  | Contactor Energize / De-energize Under PLC Control |  |  | Test ID #: |  |  |  |
|  | Description: |  | Verify the Mitsubishi SN35 contactor energizes and de-energizes correctly in response to PLC output commands. |  |  | Type: |  |  | ✓ white box □ black box □ _____________ |
| Tester Information |  |  |  |  |  |  |  |  |  |
|  | Name of Tester: |  |  |  |  | Date: |  |  |  |
|  | HW/SW Version: |  | 1.0 |  |  | Time: |  |  |  |
|  | Setup: |  | System powered, VFD at 0 Hz. PLC in RUN mode. Laptop with DirectSoft force table open. Multimeter in continuity mode. E-stop released, deadman switches released. |  |  |  |  |  |  |
| T E S T | INPUTS |  |  | EXPECTED OUTPUTS |  | P A S S | F A I L | N/A | Comments |
| 1 | PLC output to contactor coil = OFF (logic 0). Measure coil voltage and continuity across main contacts. |  |  | Coil voltage ~0 VDC. Main contacts OPEN: continuity = open circuit (inf ohm). No click. |  |  |  |  |  |
| 2 | Force PLC output = ON (logic 1) via DirectSoft force table. Observe contactor. Measure coil voltage and contact continuity. |  |  | Audible click. Contacts CLOSED: <1 ohm. Coil voltage = control supply (24 VDC or 120 VAC) |  |  |  |  |  |
| 3 | force PLC output = OFF. Observe contactor de-energize. Measure contact continuity. |  |  | Audible click. Contacts OPEN within 100 ms. Continuity = open circuit. |  |  |  |  |  |
| 4 | Repeat energize/de-energize cycle 5 times. Note any chatter, delay, or failure to close/open. |  |  | Consistent response all 5 cycles. No chatter. No visible arcing. Record: ___/5 PASS. |  |  |  |  |  |
|  | Overall test result: |  |  |  |  |  |  |  |  |